Lab 1

Advanced Biomedical Signal and Image Processing

Master: Plasturgy & Biomedical Engineering

Pr. Omar ELOUTASSI

Group:

Names:

# Objective:

What is the objective of this practical work, and how can we acquire and display biomedical signals, particularly ECG signals, using software tools like Octave?

# Tasks to Achieve

## Task 1: Record or Simulate an ECG Signal

### Simulating an ECG Signal in Octave

An ECG signal can be approximated using a sum of sinusoids or using built-in functions like ecg(N) in some toolboxes.

Copy this code and run it on GNU Octave

fs = 500;  % Sampling frequency in Hz

t = 0:1/fs:2;  % Time vector for 2 seconds

ecg_signal = sin(2*pi*1*t) + 0.5*sin(2*pi*3*t) + 0.2*sin(2*pi*50*t);

plot(t, ecg_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Simulated ECG Signal');

grid on;

screenshot your window using a sampling frequency 1000s

screenshot again the result using sampling at 500s and change sinus to cosines in the code. For both signals calculate the period in second.

## Task 2: Visualizing the ECG Signal in Time and Frequency Domains

### Time-Domain Visualization

You can visualize the same signal as above but adding a title as time domain. However change the title to 'EKG signal Time Domain'

figure;

plot(t, ecg_signal);

xlabel('Time (s)');

ylabel('ECG Amplitude');

title('ECG Signal in Time Domain');

grid on;

### Frequency-Domain Analysis Using Fourier Transform

The following code is valuable to analyze the signal as frequency domain by applying the Fourier transform

N = length(ecg_signal);

frequencies = (0:N-1)*(fs/N);

fft_ecg = abs(fft(ecg_signal));

figure;

plot(frequencies(1:N/2), fft_ecg(1:N/2));

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('ECG Signal in Frequency Domain');

grid on;

Explain why we apply such function to the signal.

# 3. Conclusion