Lab 2

Advanced Biomedical Signal and Image Processing

Master: Plasturgy & Biomedical Engineering

Pr. Omar ELOUTASSI

Group:

Names:

# 1. Objective:

What is the goal of this practical work in digital signal processing for biomedical signals?

What types of noise commonly affect biomedical signals, and how can digital filters help in removing them?

What is the differences between time domain and frequency domain?

How can we transform a signal from time domain to frequency domain?

Give some examples of extracted features from both time domain and frequency domain

# 2. Tasks to achieve: Understanding the ECG signal and noise Sources

## Task 1: Identify the frequency range of a typical ECG signal (0.05–100 Hz).

Copy this code run it and screenshot the result

clc; clear; close all;

% Define the time axis (one heartbeat duration ~0.8s)

fs = 1000; % Sampling frequency (Hz)

t = -0.5:1/fs:1.5; % Time vector (covering one full cycle)

% Define the peaks and widths of P, Q, R, S, T, and U waves

p_wave = 0.1 * exp(-((t - 0.1) / 0.025).^2);  % P wave

q_wave = -0.15 * exp(-((t - 0.2) / 0.010).^2); % Q wave

r_wave = 1.0 * exp(-((t - 0.25) / 0.015).^2);  % R wave

s_wave = -0.2 * exp(-((t - 0.3) / 0.010).^2);  % S wave

t_wave = 0.3 * exp(-((t - 0.4) / 0.050).^2);  % T wave

u_wave = 0.05 * exp(-((t - 0.6) / 0.030).^2);  % U wave

% Sum all waves to create a complete ECG cycle

ecg_signal = p_wave + q_wave + r_wave + s_wave + t_wave + u_wave;

% Repeat the ECG signal to simulate multiple heartbeats

num_beats = 5; % Number of heartbeats

full_ecg = repmat(ecg_signal, 1, num_beats);

full_t = linspace(0, num_beats, length(full_ecg));

% Plot the ECG signal

figure;

plot(full_t, full_ecg, 'b', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('Simulated ECG Signal with P, Q, R, S, T, and U Waves');

grid on;

Paste your screenshot result and analyze it

## Task 2 Identify the types of noise affecting the ECG signal:

### Task 2.1. Add a baseline Wander noise

Copy this code and run it then screenshot the result

clc; clear; close all;

% Define the time axis

fs = 1000; % Sampling frequency (Hz)

t = -0.5:1/fs:1.5; % Time vector (for one heartbeat)

% Define ECG waves using Gaussian functions

p_wave = 0.1 * exp(-((t - 0.1) / 0.025).^2);   % P wave

q_wave = -0.15 * exp(-((t - 0.2) / 0.010).^2); % Q wave

r_wave = 1.0 * exp(-((t - 0.25) / 0.015).^2);  % R wave

s_wave = -0.2 * exp(-((t - 0.3) / 0.010).^2);  % S wave

t_wave = 0.3 * exp(-((t - 0.4) / 0.050).^2);   % T wave

u_wave = 0.05 * exp(-((t - 0.6) / 0.030).^2);  % U wave

% Combine the waves to create one ECG cycle

ecg_signal = p_wave + q_wave + r_wave + s_wave + t_wave + u_wave;

% Repeat the ECG cycle multiple times to simulate multiple beats

num_beats = 5; % Number of heartbeats

full_ecg = repmat(ecg_signal, 1, num_beats);

full_t = linspace(0, num_beats, length(full_ecg));

% Simulate baseline wander (low-frequency sinusoidal drift < 0.5 Hz)

baseline_wander = 0.3 * sin(2 * pi * 0.2 * full_t); % 0.2 Hz sinusoidal drift

% Add baseline wander to the ECG signal

noisy_ecg = full_ecg + baseline_wander;

% Plot the original and noisy ECG signals

figure;

subplot(2,1,1);

plot(full_t, full_ecg, 'b', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original ECG Signal');

grid on;

subplot(2,1,2);

plot(full_t, noisy_ecg, 'r', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG Signal with Baseline Wander (0.2 Hz)');

grid on;

Paste your screenshot and analyze the result

### Task 2.2 Add a Powerline interference (50/60 Hz).

Copy this code run it and screenshot the result

clc; clear; close all;

% Define the time axis

fs = 1000; % Sampling frequency (Hz)

t = -0.5:1/fs:1.5; % Time vector (for one heartbeat)

% Define ECG waves using Gaussian functions

p_wave = 0.1 * exp(-((t - 0.1) / 0.025).^2);   % P wave

q_wave = -0.15 * exp(-((t - 0.2) / 0.010).^2); % Q wave

r_wave = 1.0 * exp(-((t - 0.25) / 0.015).^2);  % R wave

s_wave = -0.2 * exp(-((t - 0.3) / 0.010).^2);  % S wave

t_wave = 0.3 * exp(-((t - 0.4) / 0.050).^2);   % T wave

u_wave = 0.05 * exp(-((t - 0.6) / 0.030).^2);  % U wave

% Combine the waves to create one ECG cycle

ecg_signal = p_wave + q_wave + r_wave + s_wave + t_wave + u_wave;

% Repeat the ECG cycle multiple times to simulate multiple beats

num_beats = 5; % Number of heartbeats

full_ecg = repmat(ecg_signal, 1, num_beats);

full_t = linspace(0, num_beats, length(full_ecg));

% Simulate baseline wander (low-frequency sinusoidal drift < 0.5 Hz)

baseline_wander = 0.3 * sin(2 * pi * 0.2 * full_t); % 0.2 Hz sinusoidal drift

% Simulate powerline interference (sinusoidal noise at 50 Hz)

powerline_noise = 0.1 * sin(2 * pi * 50 * full_t); % 50 Hz sinusoidal noise

% Add baseline wander to the ECG signal

ecg_with_baseline_wander = full_ecg + baseline_wander;

% Add both baseline wander and powerline interference

ecg_with_noise = ecg_with_baseline_wander + powerline_noise;

% Plot the signals

figure;

subplot(3,1,1);

plot(full_t, full_ecg, 'b', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original ECG Signal');

grid on;

subplot(3,1,2);

plot(full_t, ecg_with_baseline_wander, 'r', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG Signal with Baseline Wander (0.2 Hz)');

grid on;

subplot(3,1,3);

plot(full_t, ecg_with_noise, 'k', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG Signal with Baseline Wander and Powerline Interference (50 Hz)');

grid on;

Paste your result and analyze it

### Task 2.3. Muscle artifact

Copy this code run it and analyze it

clc; clear; close all;

% Define the time axis

fs = 1000; % Sampling frequency (Hz)

t = -0.5:1/fs:1.5; % Time vector (for one heartbeat)

% Define ECG waves using Gaussian functions

p_wave = 0.1 * exp(-((t - 0.1) / 0.025).^2);   % P wave

q_wave = -0.15 * exp(-((t - 0.2) / 0.010).^2); % Q wave

r_wave = 1.0 * exp(-((t - 0.25) / 0.015).^2);  % R wave

s_wave = -0.2 * exp(-((t - 0.3) / 0.010).^2);  % S wave

t_wave = 0.3 * exp(-((t - 0.4) / 0.050).^2);   % T wave

u_wave = 0.05 * exp(-((t - 0.6) / 0.030).^2);  % U wave

% Combine the waves to create one ECG cycle

ecg_signal = p_wave + q_wave + r_wave + s_wave + t_wave + u_wave;

% Repeat the ECG cycle multiple times to simulate multiple beats

num_beats = 5; % Number of heartbeats

full_ecg = repmat(ecg_signal, 1, num_beats);

full_t = linspace(0, num_beats, length(full_ecg));

% Simulate Baseline Wander (low-frequency sinusoidal drift < 0.5 Hz)

baseline_wander = 0.3 * sin(2 * pi * 0.2 * full_t); % 0.2 Hz drift

% Simulate Powerline Interference (sinusoidal noise at 50 Hz)

powerline_noise = 0.1 * sin(2 * pi * 50 * full_t); % 50 Hz noise

% Simulate Muscle Artifacts (high-frequency random noise > 100 Hz)

muscle_artifacts = 0.05 * randn(size(full_ecg)); % Random high-freq noise

% Add baseline wander to the ECG signal

ecg_with_baseline_wander = full_ecg + baseline_wander;

% Add both baseline wander and powerline interference

ecg_with_powerline_noise = ecg_with_baseline_wander + powerline_noise;

% Add all three types of noise (baseline, powerline, and muscle artifacts)

ecg_with_all_noise = ecg_with_powerline_noise + muscle_artifacts;

% Plot the signals

figure;

subplot(4,1,1);

plot(full_t, full_ecg, 'b', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original ECG Signal');

grid on;

subplot(4,1,2);

plot(full_t, ecg_with_baseline_wander, 'r', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG with Baseline Wander (0.2 Hz)');

grid on;

subplot(4,1,3);

plot(full_t, ecg_with_powerline_noise, 'k', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG with Baseline Wander + Powerline Interference (50 Hz)');

grid on;

subplot(4,1,4);

plot(full_t, ecg_with_all_noise, 'm', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG with Baseline Wander + Powerline Interference + Muscle Artifacts');

grid on;

Paste your screenshot and analyze it

## Task 3 combine all noises

Copy this code run then screenshot the result

clc; clear; close all;

% Define sampling parameters

fs = 1000;  % Sampling frequency (Hz)

t = -0.5:1/fs:1.5; % Time vector covering one full cycle

% --- Define ECG waves using Gaussian functions ---

p_wave = 0.1 * exp(-((t - 0.1) / 0.025).^2);   % P wave

q_wave = -0.15 * exp(-((t - 0.2) / 0.010).^2); % Q wave

r_wave = 1.0 * exp(-((t - 0.25) / 0.015).^2);  % R wave (highest peak)

s_wave = -0.2 * exp(-((t - 0.3) / 0.010).^2);  % S wave

t_wave = 0.3 * exp(-((t - 0.4) / 0.050).^2);   % T wave

u_wave = 0.05 * exp(-((t - 0.6) / 0.030).^2);  % U wave

% Sum all waves to create a complete ECG cycle

ecg_cycle = p_wave + q_wave + r_wave + s_wave + t_wave + u_wave;

% Repeat the ECG cycle multiple times to simulate multiple heartbeats

num_beats = 5; % Number of heartbeats

full_ecg = repmat(ecg_cycle, 1, num_beats);

full_t = linspace(0, num_beats, length(full_ecg));

% --- Add Noise ---

% 1. Baseline Wander (low-frequency drift < 0.5 Hz)

baseline_wander = 0.3 * sin(2 * pi * 0.2 * full_t); % 0.2 Hz sinusoidal drift

% 2. Powerline Interference (50 Hz sinusoidal noise)

powerline_noise = 0.1 * sin(2 * pi * 50 * full_t);

% 3. High-frequency Muscle Artifacts (>100 Hz random noise)

muscle_artifact = 0.05 * randn(size(full_t));

% Combine all noise sources

noisy_ecg = full_ecg + baseline_wander + powerline_noise + muscle_artifact;

% Plot Original ECG

subplot(3,1,1);

plot(full_t, full_ecg, 'b', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original ECG Signal');

grid on;

% Plot Noisy ECG

subplot(3,1,2);

plot(full_t, noisy_ecg, 'r', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('Amplitude');

title('ECG Signal with Noise (Baseline Wander, Powerline, and Muscle Artifacts)');

grid on;

Screen shot the result and analyze it

## Task 4

Put comments to this  this code (explain the signification)

fs = 500;

T = 3;

t = 0:1/fs:T;

## Task 4.1, simulate an ECG signal

Copy this code then run it

% Define heartbeat frequency

HR = 60;  % Heart rate (bpm)

f = HR / 60; % Convert to Hz (cycles per second)

% ECG wave components using Gaussian pulses

P_wave = 0.1 * exp(-((t - 0.2) / 0.05).^2);

Q_wave = -0.15 * exp(-((t - 0.3) / 0.02).^2);

R_wave = 1.0 * exp(-((t - 0.32) / 0.01).^2);

S_wave = -0.25 * exp(-((t - 0.35) / 0.02).^2);

T_wave = 0.2 * exp(-((t - 0.6) / 0.1).^2);

% Sum the components to form the ECG cycle

ECG_cycle = P_wave + Q_wave + R_wave + S_wave + T_wave;

% Create 3 cycles by repeating the waveform

ECG_signal = repmat(ECG_cycle, 1, 3);

% Adjust time vector accordingly

t = linspace(0, 3, length(ECG_signal));

% Plot ECG signal

figure;

plot(t, ECG_signal, 'k');

xlabel('Time (s)');

ylabel('Amplitude');

title('Simulated ECG Signal (3 Heartbeats)');

grid on;

Extract the typical ECG wave characteristics in the table bellow

| Wave | Amplitude (mV) | Duration (s) | Description |
| --- | --- | --- | --- |
| P |  |  |  |
| Q |  |  |  |
| R |  |  |  |
| S |  |  |  |
| T |  |  |  |

## Task 4.2, add a noise

clc; clear; close all;

fs = 500;        % Sampling frequency (Hz)

T = 3;           % Duration (seconds)

t = 0:1/fs:1;    % One heartbeat duration (1s for 60 BPM)

% ECG wave components (fine-tuned)

P_wave = 0.15 * exp(-((t - 0.2) / 0.045).^2); % P wave

Q_wave = -0.2 * exp(-((t - 0.3) / 0.015).^2); % Q wave

R_wave = 1.0 * exp(-((t - 0.32) / 0.012).^2); % R wave

S_wave = -0.3 * exp(-((t - 0.35) / 0.015).^2); % S wave

T_wave = 0.35 * exp(-((t - 0.62) / 0.1).^2);  % T wave

% Summing the waves to form a single ECG heartbeat

ECG_cycle = P_wave + Q_wave + R_wave + S_wave + T_wave;

% Repeat for 3 heartbeats

ECG_signal = repmat(ECG_cycle, 1, 3);

% Adjust time vector for 3 seconds

t = linspace(0, 3, length(ECG_signal));

% --- Adding Noise ---

% 1. Baseline Wander (low-frequency sinusoidal drift, simulating respiration)

baseline_wander = 0.1 * sin(2 * pi * 0.5 * t);  % 0.5 Hz slow drift

% 2. Powerline Interference (50 Hz noise)

powerline_noise = 0.05 * sin(2 * pi * 50 * t);

% 3. White Gaussian Noise (simulating muscle noise)

gaussian_noise = 0.02 * randn(size(t));

% Combine ECG signal with noise

ECG_noisy = ECG_signal + baseline_wander + powerline_noise + gaussian_noise;

% Plot ECG signal

figure;

plot(t, ECG_noisy, 'k', 'LineWidth', 1.2);

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Simulated ECG Signal with Noise (3 Heartbeats)');

grid on;

axis([0 3 -0.7 1.2]); % Set axis limits for clarity

What are the noise we added at the original signal?

You can use this code form more details

% --- Adding Noise ---

% 1. Baseline Wander (low-frequency sinusoidal drift, simulating respiration)

baseline_wander = 0.1 * sin(2 * pi * 0.5 * t);  % 0.5 Hz slow drift

% 2. Powerline Interference (50 Hz noise)

powerline_noise = 0.05 * sin(2 * pi * 50 * t);

% 3. White Gaussian Noise (simulating muscle noise)

gaussian_noise = 0.02 * randn(size(t));

figure;

plot(t, baseline_wander, 'r', 'LineWidth', 1); hold on;

plot(t, powerline_noise, 'b', 'LineWidth', 1);

plot(t, gaussian_noise, 'g', 'LineWidth', 1);

legend('Baseline Wander', 'Powerline Noise (50 Hz)', 'White Gaussian Noise');

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Added Noise Components');

grid on;

## Task 5, filter the signal

## Task 5.1, add one noise (power line noise)

To make it easy we will add just one noise at the simulated ECG signal, you can use this code

clc; clear; close all;

fs = 500;        % Sampling frequency (Hz)

T = 3;           % Duration (seconds)

t = 0:1/fs:1;    % One heartbeat duration (1s for 60 BPM)

% ECG wave components (fine-tuned)

P_wave = 0.15 * exp(-((t - 0.2) / 0.045).^2); % P wave

Q_wave = -0.2 * exp(-((t - 0.3) / 0.015).^2); % Q wave

R_wave = 1.0 * exp(-((t - 0.32) / 0.012).^2); % R wave

S_wave = -0.3 * exp(-((t - 0.35) / 0.015).^2); % S wave

T_wave = 0.35 * exp(-((t - 0.62) / 0.1).^2);  % T wave

% Summing the waves to form a single ECG heartbeat

ECG_cycle = P_wave + Q_wave + R_wave + S_wave + T_wave;

% Repeat for 3 heartbeats

ECG_signal = repmat(ECG_cycle, 1, 3);

% Adjust time vector for 3 seconds

t = linspace(0, 3, length(ECG_signal));

% --- Adding Noise ---

% 2. Powerline Interference (50 Hz noise)

powerline_noise = 0.05 * sin(2 * pi * 50 * t);

% Combine ECG signal with noise

ECG_noisy = ECG_signal + powerline_noise;

% Plot ECG signal

figure;

plot(t, ECG_noisy, 'k', 'LineWidth', 1.2);

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Simulated ECG Signal with Noise (3 Heartbeats)');

grid on;

axis([0 3 -0.7 1.2]); % Set axis limits for clarity

## Task 5.2, filter the noise

We can design a low pass filter for this task use the flowing code

clc; clear; close all;

pkg load signal;  % Load the signal processing package

graphics_toolkit("gnuplot"); % Ensure plotting works in Octave

fs = 500;        % Sampling frequency (Hz)

T = 3;           % Duration (seconds)

t = 0:1/fs:1;    % One heartbeat duration (1s for 60 BPM)

% --- ECG Wave Components ---

P_wave = 0.15 * exp(-((t - 0.2) / 0.045).^2);

Q_wave = -0.2 * exp(-((t - 0.3) / 0.015).^2);

R_wave = 1.0 * exp(-((t - 0.32) / 0.012).^2);

S_wave = -0.3 * exp(-((t - 0.35) / 0.015).^2);

T_wave = 0.35 * exp(-((t - 0.62) / 0.1).^2);

% --- Summing the waves ---

ECG_cycle = P_wave + Q_wave + R_wave + S_wave + T_wave;

ECG_signal = repmat(ECG_cycle, 1, 3); % Repeat for 3 heartbeats

t = linspace(0, 3, length(ECG_signal)); % Adjust time vector

% --- Adding Noise ---

powerline_noise = 0.05 * sin(2 * pi * 50 * t); % 50 Hz interference

ECG_noisy = ECG_signal + powerline_noise;

% --- Low-Pass Filter Design ---

fc = 40;  % Cutoff frequency (Hz) - Keeps important ECG features

order = 4; % Filter order

[b, a] = butter(order, fc / (fs / 2), 'low'); % Low-pass Butterworth filter

% --- Apply the Low-Pass Filter ---

ECG_filtered = filter(b, a, ECG_noisy);

% --- Plot Results ---

figure(1);

subplot(2,1,1);

plot(t, ECG_noisy, 'r', 'LineWidth', 1.2);

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Noisy ECG Signal (with 50 Hz interference)');

grid on;

axis([0 3 -0.7 1.2]);

subplot(2,1,2);

plot(t, ECG_filtered, 'b', 'LineWidth', 1.2);

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Filtered ECG Signal (Low-Pass Filter Applied)');

grid on;

axis([0 3 -0.7 1.2]);

drawnow;  % Force plot update in Octave

Compare the filtered signal and the noisy signal

## Task 6, perform an FFT to the signal

You can use this code

clc; clear; close all;

pkg load signal;  % Load the signal processing package

graphics_toolkit("gnuplot"); % Ensure plotting works in Octave

fs = 500;        % Sampling frequency (Hz)

T = 3;           % Duration (seconds)

t = 0:1/fs:1;    % One heartbeat duration (1s for 60 BPM)

% --- ECG Wave Components ---

P_wave = 0.15 * exp(-((t - 0.2) / 0.045).^2);

Q_wave = -0.2 * exp(-((t - 0.3) / 0.015).^2);

R_wave = 1.0 * exp(-((t - 0.32) / 0.012).^2);

S_wave = -0.3 * exp(-((t - 0.35) / 0.015).^2);

T_wave = 0.35 * exp(-((t - 0.62) / 0.1).^2);

% --- Summing the waves ---

ECG_cycle = P_wave + Q_wave + R_wave + S_wave + T_wave;

ECG_signal = repmat(ECG_cycle, 1, 3); % Repeat for 3 heartbeats

t = linspace(0, 3, length(ECG_signal)); % Adjust time vector

% --- Adding Noise ---

powerline_noise = 0.05 * sin(2 * pi * 50 * t); % 50 Hz interference

ECG_noisy = ECG_signal + powerline_noise;

% --- Low-Pass Filter Design ---

fc = 40;  % Cutoff frequency (Hz) - Keeps important ECG features

order = 4; % Filter order

[b, a] = butter(order, fc / (fs / 2), 'low'); % Low-pass Butterworth filter

% --- Apply the Low-Pass Filter ---

ECG_filtered = filter(b, a, ECG_noisy);

% --- FFT Analysis ---

N = length(ECG_noisy);          % Length of the signal

f = (0:N-1) * fs / N;           % Frequency vector

ECG_noisy_fft = abs(fft(ECG_noisy));  % FFT of noisy signal

ECG_filtered_fft = abs(fft(ECG_filtered));  % FFT of filtered signal

% --- Plot Time Domain and Frequency Domain ---

figure;

% Time Domain Plot

subplot(2,1,1);

plot(t, ECG_noisy, 'r', 'LineWidth', 1.2);

hold on;

plot(t, ECG_filtered, 'b', 'LineWidth', 1.2);

xlabel('Time (s)');

ylabel('Amplitude (mV)');

title('Noisy ECG vs Filtered ECG in Time Domain');

legend('Noisy ECG', 'Filtered ECG');

grid on;

axis([0 3 -0.7 1.2]);

% Frequency Domain Plot

subplot(2,1,2);

plot(f, ECG_noisy_fft, 'r', 'LineWidth', 1.2);

hold on;

plot(f, ECG_filtered_fft, 'b', 'LineWidth', 1.2);

xlabel('Frequency (Hz)');

ylabel('Amplitude');

title('Frequency Spectrum of Noisy and Filtered ECG');

legend('Noisy ECG FFT', 'Filtered ECG FFT');

xlim([0 100]); % Focus on the relevant frequency range

grid on;

drawnow;  % Force plot update in Octave

Which figure shows the frequency domain?

Does the bottom plot show the frequency spectrum for both the noisy and the filtered signals?

Do we limit the x-axis to 100 Hz to zoom in on the frequency range of interest (e.g., powerline noise at 50 Hz)?

Does the top plot show both the noisy and filtered signals in the time domain?

In the time domain, can you observe the difference between the noisy and filtered ECG signals?

In the frequency domain, can you identify the powerline interference at 50 Hz in the noisy signal (visible as a spike at 50 Hz)? After filtering, should this spike be reduced or removed, confirming the effectiveness of the low-pass filter?

Does this analysis allow you to understand how the low-pass filter reduces high-frequency noise and preserves the important components of the ECG signal?

# Conclusion