Parcours Électronique S6 Optoélectronique

Lab 2: Responsivity and quantum efficiency

Eloutassi Omar

2025-2026

Student Names:

Groupe

# Purpose:

Read the entire text and complete the assigned tasks. Ensure you answer every question highlighted in orange, and then conclude by stating the purpose of this lab.

# Tasks

To begin your semiconductor lab in ISIS Proteus, follow these steps to open the software and set up your workspace:

Launch the Application: Click the Start button on your Windows taskbar, navigate to the Proteus 8 Professional folder, and select the Proteus application.

Start a New Project: On the Proteus home page, click the New Project icon. Use the New Project Wizard to:

Enter a Project Name and choose a save directory.

Select Create a schematic from the selected template and choose the DEFAULT (A4) layout.

Choose Do not create a PCB layout and No Firmware Project as this lab focuses purely on circuit simulation.

Enter Schematic Capture: Click Finish to enter the main design area (the Schematic Capture window), where you will build your circuit

## Task 1: Responsivity and quantum efficiency

This simulation demonstrates how light power converts to current and how carrier lifetime affects gain. Explain the last process. Give the necessary formula based on your tutorial 3

### Component Selection:

Photodiode: Use a generic Photodiode model or an IR Sensor element. Operational Amplifier: Use an LM358 or TL081 for the TIA stage. DC Power Supply: 5V to 12V for biasing. Torch LDR. LDR. Virtual Instruments: DC Ammeter (to measure the current flow from the photodiode) and DC Voltmeter.

Screen shot your schema and paste in the text

### Simulation Task:

Reverse Bias: Connect the photodiode in reverse bias to a DC source.

TIA Configuration: Connect the photodiode anode to the inverting input of the Op-Amp. Place a feedback resistor (Rfbetween the output and inverting input.

### Measurement:

Place a DC Ammeter in series with the photodiode to measure Output Current Ip.

Represent "Incident Optical Power Pin by varying the light intensity parameter in the photodiode properties (Torch or equivalent).

Verification: Calculate Responsivity by dividing the ammeter reading Ip using your choosing Pin value, you can assume that Pin=Imeasured0.48=Ip0.48

## Task 2:

Replace the Torch LDR by an active LDR and paste your schema bellow

## Conclusion:

Please submit your lab report to o.eloutassi@umi.ac.ma using only your academic email address; submissions from personal accounts will not be accepted.

Use the subject line Lab 2 2526 [Your Group]. You do not need to include a message in the body of the email unless you are responding to a specific task. Save and rename your file as [YourName][YourGroup]Lab22526.doc before attaching it. You should attached in the message your project.